External Communication Protocol

Version: B12.1

Date: 2005/02/04

External communication protocol include two parts：A. setup connecting, B. send command.

BenQ default Serial Port :

Baud Rate:	19200

Parity:		none

Data bits:		8

Stop bits:		1

Flow Control:	none

- Setup Connecting

A typical Packet transaction session is shown in Figure 1

PC                   BenQ PB8250

Figure 1 ### Host System Target System

Packet to Target => <= ACK <= Packet to Host Packet to Target => <= ACK <= Packet to Host Packet to Target =>

d

b

a

c

e

a. 1st Packet to Target (BenQ PB8250) structure like as below (Table 1)

| Packet Header | Byte0 | 0xBE | Magic Number |
| --- | --- | --- | --- |
|  | Byte1 | 0xEF |  |
|  | Byte2 | 0x01 | Packet Type |
|  | Byte3 | 0x05 | Packet size (Low) |
|  | Byte4 | 0x00 | Packet size (High) |
|  | Byte5 | 0xD1 | CRC (Low) |
|  | Byte6 | 0xFA | CRC (High) |
| # Packet Payload | Byte7 | 0x01 | System Info Type |
|  | Byte8 | 0x02 | Version Number |
|  | Byte9 | 0x00 |  |
|  | Byte10 | 0x00 | Object ID |
|  | Byte11 | 0x00 | Level |

Table 1

b. The Ack of Packet to Host (PC) (Table 2)

| Ack | Byte0 | 0x1E | PAK |
| --- | --- | --- | --- |
| Packet Header | Byte1 | 0xBE | Magic Number |
|  | Byte2 | 0xEF |  |
|  | Byte3 | 0x01 | Packet Type |
|  | Byte4 | 0x05 | Packet size (Low) |
|  | Byte5 | 0x00 | Packet size (High) |
|  | Byte6 | 0xD1 | CRC (Low) |
|  | Byte7 | 0xFA | CRC (High) |
| # Packet Payload | Byte8 | 0x01 | System Info Type |
|  | Byte9 | 0x02 | Version Number |
|  | Byte10 | 0x00 |  |
|  | Byte11 | 0x00 | Object ID |
|  | Byte12 | 0x00 | Level |

Table 2

PAK means that PC will follow the received Packet data

c. Packet same as 1st Packet (Table 1)

d. Same as Ack (Table 2)

e. Packet to Target (BenQ PB8250) structure (Table 3)

| Packet Header | Byte0 | 0xBE | Magic Number |
| --- | --- | --- | --- |
|  | Byte1 | 0xEF |  |
|  | Byte2 | 0x01 | Packet Type |
|  | Byte3 | 0x05 | Packet size (Low) |
|  | Byte4 | 0x00 | Packet size (High) |
|  | Byte5 | 0xA9 | CRC (Low) |
|  | Byte6 | 0xC6 | CRC (High) |
| # Packet Payload | Byte7 | 0x00 | System Info Type |
|  | Byte8 | 0x00 | Version Number |
|  | Byte9 | 0x00 |  |
|  | Byte10 | 0x00 | Object ID |
|  | Byte11 | 0x00 | Level |

Table 3

- Send Command

  - Introduction

Command packets consist of “Header” and “Payload”. The Packet Header is consistent for all packets. The Packet Payload type and content varies based on the type of packet sent. The entire packet size is variable, being the sum of the fixed-size Packet Header and variable-sized Packet Payload.

| Packet Header (fixed size) | Packet Payload (variable size) |
| --- | --- |

## Figure 2 Packet Format

Packet Header Format

All Packets use the same Packet Header format illustrated Figure 3.

| #### Byte 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Magic Number |  | Type | Packet Payload Size |  | CRC |  |
| 0xBE | 0xEF | type | size_lo | size_hi | crc_lo | crc_hi |

## Figure 3 Packet Format

The Packet Header size is fixed at seven bytes (Intel byte ordering is used). The following code fragments are taken from these source files

The Packet Header definition is shown below:

typedef struct

{

BYTE ePacketType; // type of the payload

WORD nPacketSize; // size of the payload

WORD nCRCPacket; // CRC for the entire packet

} PACKET_HEADER;

Magic Number

The Magic Number is a fixed value that is used to insure packet alignment if there are partial packets received or bytes lost. The Magic Number is a WORD in length (2 bytes). The Magic Number value is 0xEFBE. Because Intel byte ordering is used, the ls-byte of the word is sent first (byte0 = 0xBE), then the ms-byte (byte1 = 0xEF).

Packet Type

The Packet Type (ePacketType) is a BYTE in length number that defines the type of data in the packet. The following entries are valid packet typess:

| Table 4 Packet Types Packet Type Name | Packet Type Number | Description |
| --- | --- | --- |
| pt_INVALID | 0 | Invalid Packet Type |
| RESERVED | 1 | RESERVED |
| pt_EVENT | 2 | Host can send any event defined in BenQ PB8250 software. |
| pt_OPERATION | 3 | Host can send any operation defined in BenQ PB8250software. |

Packet Payload Size

The Packet Payload Size (nPacketSize) is a BYTE that defines the size of the Payload portion of the packet. If the packet contains only header information, this is zero. Therefore, the total byte count of any packet = nPacketSize plus 7 (since the Packet Header is seven bytes long).

Packet Checksum (CRC)

Each packet is CRC’ed using the tables later in this document. This number is the CRC value for the complete packet including the Packet Header and Packet Payload. The CRC is calculated with the nCRCPacket value initialized to zero.

  - Packet Payload Definition

Event Packet Type

The Event packet is used by the host system to send virtual events (such as Zoom, Source, Auto Adjust, etc.) to the target system. Packet payload size is 6 bytes.

| Byte | Field Name | Field Value | Description |
| --- | --- | --- | --- |
| 0-1 | Virtual Event |  | Virtual Event ID as defined through Configurator |
| 2-5 | Parameter |  | Parameter that can be associated with the event. |

. Table 5 Event Packet Type Format

The source code definition of the Message packet data structure is:

typedef struct

{

WORD      eEvent;

DWORD    dwParam;

} EVENT_MESSAGE;

This lets you send any event defined in Configurator to the system including all remote, IR, or special events

Operation Packet Type

The Operation packet is used by the host system to execute operations (such as Brightness, Contrast, Image Position, etc) in the target system. The Operation packet payload size is 25 bytes.

| Byte | Field Name | Field Value | Description |
| --- | --- | --- | --- |
| 0 | Operation Type | 1 | OPERATION_SET |
|  |  | 2 | OPERATION_GET |
|  |  | 3 | OPERATION_INCREMENT |
|  |  | 4 | OPERATION_DECREMENT |
|  |  | 5 | OPERATION_EXECUTE |
| 1-2 | Operation |  | Operation ID as defined in Configurator |
| 3-4 | Is Avail |  | Operation is available |
| 5-8 | Operation Target |  | Used for Operation with Targets. These Targets are defined in configurator. For instance, op_BRIGHTNESS has a Target of either MAIN or PIP window.. |
| 9-12 | Operation Value |  | Value of the Set on a set or the Value of the Get on a Return. |
| 13-16 | Operation Value of minimum. |  | The Minimum Value of the set for operation command. |
| 17-20 | Operation Value of maximum |  | The Maximum Value of the set for operation command. |
| 21-24 | Operation Value of Increment |  | The Increment Value of the set for operation command. |

Table 6 Operation Packet Payload Format

The source code definition of the Operation packet data structure is:

typedef struct

{

eOPERATION_TYPE   eOpType;

WORD      eOperation;

WORD      bisAvail;

DWORD     dwTarget;

DWORD     dwValue;

DWORD     lmMin;

DWORD     lmMax;

DWORD     lmInc;

} OPERATION_MESSAGE;

This lets the user directly perform logical operations such as “Set Contrast = 80”.

3. Send Command

PC                   BenQ PB8250

### Host System Target System

Packet to Target => <= ACK

a

b

Figure 4

- The structure of Command (EX. input select) send to Target (BenQ PB8250) like as below (Table 7)

| Packet Header | Byte0 | 0xBE | Magic Number |
| --- | --- | --- | --- |
|  | Byte1 | 0xEF |  |
|  | Byte2 | 0x02 | Packet Type |
|  | Byte3 | 0x06 | Packet size (Low) |
|  | Byte4 | 0x00 | Packet size (High) |
|  | Byte5 | 0x80 | CRC (Low) |
|  | Byte6 | 0xC7 | CRC (High) |
| # Packet Payload | Byte7 | 0xC9 | Virtual Event ID |
|  | Byte8 | 0x00 |  |
|  | Byte9 | 0x00 | Parameter |
|  | Byte10 | 0x00 |  |
|  | Byte11 | 0x00 |  |
|  | Byte12 | 0x00 |  |

Table 7

- Target return to Host (PC) Ack like as below Table 8

| Ack | Byte0 | 0x06 | ACK |
| --- | --- | --- | --- |

Table 8

C.  Serial Communication Cable and Parameters

For external serial communication from a computer to BenQ projector, BenQ recommends manufactures use RS-232 communications over a straight through serial cable a 9 pin female D-sub9 connector.

The standard D-sub9 connector on the computer is a male connector, and BenQ projector, too. The wiring between the computer and BenQ projector is a straight through cable. A 9 pin female to 9 pin female straight through cable is a very standard part and readily available in many lengths.

Female D-sub9 pin-out numbering and definitions on both terminal :

| Pin number | Name |
| --- | --- |
| 2 | Transmit |
| 3 | Receive |
| 5 | Ground |

PW Serial uses the following default serial port settings:

Baud Rate: 19200

Parity: none

Data bits: 8

Stop bits: 1

Flow Control: none

D. Software Flow Chart

Build serial communication port

Baud rate: 19200

Parity: 	 none

Data bits: 	8

Stop bits: 	1

Transmit 1st Packet (see Table 1)

Delay 100ms

Transmit 2nd Packet (see Table1)

Delay 100ms

Transmit 3rd Packet (see Table3)

Transmit Command (see Table7)

# Command List

## Event Packet Type command:

| Command | Packet Header (7 bytes) | Packet Payload (6 bytes) |
| --- | --- | --- |
| Power | BE EF 02 06 00 E6 C7 | CF 00 00 00 00 00 |
| Auto | BE EF 02 06 00 07 CD | BE 00 00 00 00 00 |
| Input select | BE EF 02 06 00 34 CD | BD 00 00 00 00 00 |
| Menu | BE EF 02 06 00 D6 CC | BF 00 00 00 00 00 |
| Exit | BE EF 02 06 00 9D C6 | C4 00 00 00 00 00 |
| Zoom + | BE EF 02 06 00 DC C5 | D5 00 00 00 00 00 |
| Zoom - | BE EF 02 06 00 EF C5 | D6 00 00 00 00 00 |
| Vol + | BE EF 02 06 00 6B C4 | D2 00 00 00 00 00 |
| Vol - | BE EF 02 06 00 BA C5 | D3 00 00 00 00 00 |
| PIP Source | BE EF 02 06 00 31 C1 | E8 00 00 00 00 00 |
| Freeze | BE EF 02 06 00 89 C5 | D0 00 00 00 00 00 |
| Ratio | BE EF 02 06 00 CE C1 | E7 00 00 00 00 00 |
| RS232 Power ON | BE EF 02 06 00 38 C3 | F1 00 00 00 00 00 |
| RS232 Power OFF | BE EF 02 06 00 0B C3 | F2 00 00 00 00 00 |
| Mute | BE EF 02 06 00 58 C4 | D1 00 00 00 00 00 |
| Blank | BE EF 02 06 00 0D C4 | D4 00 00 00 00 00 |

## Operation Packet Type Commands

### General Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Select PC 1 | BE EF 03 19 00 C8 4B | 01 DA 02 CC CC 00×8 CC×12 |
| Select PC 2 | BE EF 03 19 00 58 8A | 01 DA 02 CC CC 00×4 01 00 00 00 CC×12 |
| Select DVI | BE EF 03 19 00 A9 CA | 01 DA 02 CC CC 00×4 02 00 00 00 CC×12 |
| Select YPbPr | BE EF 03 19 00 0B 49 | 01 DA 02 CC CC 00×4 04 00 00 00 CC×12 |
| Select S-Video | BE EF 03 19 00 6A C8 | 01 DA 02 CC CC 00×4 06 00 00 00 CC×12 |
| Select Video | BE EF 03 19 00 FA 09 | 01 DA 02 CC CC 00×4 07 00 00 00 CC×12 |
| English Language | BE EF 03 19 00 01 DB | 01 0B 02 CC CC FF FF FF FF 00 00 00 00 CC×12 |
| Keystone + | BE EF 03 19 00 AB E7 | 03 65 02 CC CC 00 00 00 00 CC×16 |
| Keystone - | BE EF 03 19 00 C5 4D | 04 65 02 CC CC 00 00 00 00 CC×16 |

### Power Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Power ON | BE EF 03 19 00 F8 3C | 05 C3 02 CC CC FF FF FF FF 00 00 00 00 CC×12 |
| Power Off | BE EF 03 19 00 6D 61 | 05 C2 02 CC CC FF FF FF FF 00 00 00 00 CC×16 |

### PC Picture Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Brightness + | BE EF 03 19 00 44 A0 | 03 C7 02 CC CC 00 00 00 00 CC×16 |
| Brightness - | BE EF 03 19 00 2A 0A | 04 C7 02 CC CC 00 00 00 00 CC×16 |
| Contrast + | BE EF 03 19 00 2E 19 | 03 C5 02 CC CC 00 00 00 00 CC×16 |
| Contrast - | BE EF 03 19 00 40 B3 | 04 C5 02 CC CC 00 00 00 00 CC×16 |

### YPbPr Picture Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Brightness + | BE EF 03 19 00 7B 14 | 03 D9 02 CC CC FF FF FF FF CC×16 |
| Brightness - | BE EF 03 19 00 15 BE | 04 D9 02 CC CC FF FF FF FF CC×16 |
| Contrast + | BE EF 03 19 00 FA 6A | 03 F1 02 CC CC FF FF FF FF CC×16 |
| Contrast - | BE EF 03 19 00 94 C0 | 04 F1 02 CC CC FF FF FF FF CC×16 |

### S-Video / Composite Video Picture Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Brightness + | BE EF 03 19 00 E9 18 | 03 35 02 CC CC 00 00 00 00 CC x16 |
| Brightness - | BE EF 03 19 00 87 B2 | 04 35 02 CC CC 00 00 00 00 CC x16 |
| Contrast + | BE EF 03 19 00 16 FC | 03 36 02 CC CC 00 00 00 00 CC x16 |
| Contrast - | BE EF 03 19 00 78 56 | 04 36 02 CC CC 00 00 00 00 CC x16 |
| Color + | BE EF 03 19 00 83 A1 | 03 37 02 CC CC 00 00 00 00 CC X16 |
| Color - | BE EF 03 19 00 ED 0B | 04 37 02 CC CC 00 00 00 00 CC x16 |
| Tint + | BE EF 03 19 00 00 0F | 03 4A 02 CC CC 00 00 00 00 CC x16 |
| Tint - | BE EF 03 19 00 6E A5 | 04 4A 02 CC CC 00 00 00 00 CC x16 |
| Sharpness + | BE EF 03 19 00 43 D0 | 03 38 02 CC CC 00 00 00 00 CC x16 |
| Sharpness - | BE EF 03 19 00 2D 74 | 04 38 02 CC CC 00 00 00 00 CC x16 |

### Misc Controls

| Command | Packet Header (7 bytes) | Packet Payload (25 bytes) |
| --- | --- | --- |
| Color Temp –50 (0) | BE EF 03 19 00 C7 42 | 01 53 02 CC CC 00 00 00 00 CE FF FF FF CC CC CC CC CC CC CC CC CC CC CC CC |
| 0 (10) | BE EF 03 19 00 36 02 | 01 53 02 CC CC 00 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| 50 (20) | BE EF 03 19 00 ED 3C | 02 53 02 CC CC 00 00 00 00 32 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |

### PIP Controls

| PIP Size |  |  |
| --- | --- | --- |
| Off | BE EF 03 19 00 15 02 | 01 8C 02 CC CC 01 00 00 00 03 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Small | BE EF 03 19 00 E4 42 | 01 8C 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Medium | BE EF 03 19 00 74 83 | 01 8C 02 CC CC 01 00 00 00 01 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Large | BE EF 03 19 00 85 C3 | 01 8C 02 CC CC 01 00 00 00 02 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Position |  |  |
| Upper-Left | BE EF 03 19 00 1D 66 | 01 43 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Upper-Center | BE EF 03 19 00 8D A7 | 01 43 02 CC CC 01 00 00 00 01 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Upper-right | BE EF 03 19 00 7C E7 | 01 43 02 CC CC 01 00 00 00 02 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Mid-Left | BE EF 03 19 00 EC 26 | 01 43 02 CC CC 01 00 00 00 03 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Mid-Center | BE EF 03 19 00 DE 64 | 01 43 02 CC CC 01 00 00 00 04 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Mid-Right | BE EF 03 19 00 4E A5 | 01 43 02 CC CC 01 00 00 00 05 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Lower-Left | BE EF 03 19 00 BF E5 | 01 43 02 CC CC 01 00 00 00 06 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Lower-Center | BE EF 03 19 00 2F 24 | 01 43 02 CC CC 01 00 00 00 07 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Lower-Right | BE EF 03 19 00 DB 61 | 01 43 02 CC CC 01 00 00 00 08 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Source |  |  |
| S-Video | BE EF 03 19 00 E8 36 | 01 DA 02 CC CC 01 00 00 00 03 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| Video | BE EF 03 19 00 16 DD | 01 39 02 CC CC 01 00 00 00 04 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Brightness -50 (48) | BE EF 03 19 00 FE 0B | 01 35 02 CC CC 01 00 00 00 CE FF FF FF CC CC CC CC CC CC CC CC CC CC CC CC |
| 0 (126) | BE EF 03 19 00 8B CB | 01 35 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| 50 (204) | BE EF 03 19 00 FE 5E | 01 35 02 CC CC 01 00 00 00 32 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Contrast -50 (58) | BE EF 03 19 00 01 EF | 01 36 02 CC CC 01 00 00 00 CE FF FF FF CC CC CC CC CC CC CC CC CC CC CC CC |
| 0 (131) | BE EF 03 19 00 74 2F | 01 36 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| 50 (204) | BE EF 03 19 00 01 BA | 01 36 02 CC CC 01 00 00 00 32 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Color –50 (129) | BE EF 03 19 00 94 B2 | 01 37 02 CC CC 01 00 00 00 CE FF FF FF CC CC CC CC CC CC CC CC CC CC CC CC |
| 0 (157) | BE EF 03 19 00 E1 72 | 01 37 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| 50 (185) | BE EF 03 19 00 94 E7 | 01 37 02 CC CC 01 00 00 00 32 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| PIP Tint -50 (0) | BE EF 03 19 00 17 1C | 01 4A 02 CC CC 01 00 00 00 CE FF FF FF CC CC CC CC CC CC CC CC CC CC CC CC |
| 0 (128) | BE EF 03 19 00 62 DC | 01 4A 02 CC CC 01 00 00 00 00 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |
| 50 (255) | BE EF 03 19 00 17 49 | 01 4A 02 CC CC 01 00 00 00 32 00 00 00 CC CC CC CC CC CC CC CC CC CC CC CC |

## Status Query Commands

| Status Item | HEX sent to the projector (32 bytes) HEX return from the projector (32 bytes) (Query item inversed, INTEL HEX format) |
| --- | --- |
| Input Source | (to projector) BE EF 03 19 00 83 B4 02 DA 02 CC CC 00×8 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 DA 02 01 00×5 01 00 00 00 CC×12 |
| Lamp Hour | (to projector) BE EF 03 19 00 7A B1 02 07 02 CC CC FF×4 00×4 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 07 02 01 00 FF×4 AC 10 00 00 CC×12 |
| Power Status Off: 0 On: 1 Cooling: 2 | (to projector) BE EF 03 19 00 58 97 02 23 03 CC CC FF×4 00×4 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 23 03 01 00 FF×4 00 00 00 00 CC×12 |
| PC Brightness | (to projector) BE EF 03 19 00 BC B1 02 C7 02 CC CC 00×8 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 C7 02 01 00×5 20 00 00 00 CC×12 |
| PC Contrast | (to projector) BE EF 03 19 00 D6 08 02 C5 02 CC CC 00×8 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 C5 02 01 00×5 20 00 00 00 CC×12 |
| Video Brightness | (to projector) BE EF 03 19 00 11 09 02 35 02 CC CC 00×8 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 35 02 00 00×5 20 00 00 00 CC×12 |
| Video Contrast | (to projector) BE EF 03 19 00 EE ED 02 36 02 CC CC 00×8 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 36 02 00 00×5 20 00 00 00 CC×12 |
| YPbPr Brightness | (to projector) BE EF 03 19 00 83 05 02 D9 02 CC CC FF×4 00×4 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 D9 02 01 00 FF×4 80 00 00 00 CC×12 |
| YPbPr Contrast | (to projector) BE EF 03 19 00 02 7B 02 F1 02 CC CC FF×4 00×4 CC×12 (from projector) 1E BE EF 03 19 00 00 00 02 F1 02 01 00 FF×4 A9 00 00 00 CC×12 |