Projector Management Application

Version 3.0

Instruction Guide

Hitachi, Ltd.

Contents

1	Introduction	3

1.1	Outline	3

1.2	System	3

2	Installation	4

2.1	System Requirements	4

2.2	Projector Management Application Installation	4

3	Operating Instructions	7

3.1	Start Application	7

3.2	Quit Application	7

3.3	Main Screen	8

3.4	Add a group of projectors	15

3.5	Add an individual projector	15

3.6	Delete a group or individual projector	15

3.7	Change a group or individual projector settings	15

3.8	Update status	16

3.9	Turn on the power	16

3.10	Turn off the power	16

3.11	Activate monitoring scheduled events	16

3.12 	Stop monitoring scheduled events	16

3.13	Polling Interval Period	16

3.14	Mail Setting	17

3.15	About Dialogue	18

4	Mail Feature	19

4.1	Mail Format	19

4.2	Retry Processing	19

4.3	Others	19

5	Scheduling Feature	20

5.1	Event Type	20

5.2	Add an Event	21

5.2	Add an Event	21

5.3	Delete an Event	21

- Microsoft and Windows is the registered trademark of Microsoft Corporation in the US and other countries.

- Pentium is the registered trademark of Intel Corporation in the US and other countries.

- All other trademarks are the property of their respected owners.

Warning

- This software is free via download from Hitachi. The manufacturer has no responsibility for any errors that may occur while using this software.

- This software, or any product used with this software may not be used for any act which obstructs the Maintenance of the International Peace and Safety.

# 1	Introduction

## 1.1	Outline

The Projector Management Application is an application that allows you to monitor, control and diagnose projectors that are connected through a network or RS-232C. With this application, a single computer can control and monitor multiple projectors that are connected to the network or RS-232C.

## 1.2	System

The following diagram shows a typical network diagram for the Management Application.

HUB

LAN

COM2

COM1

Ethernet

/RS-232C

Converter

Port1

Port2

- Use a UTP cable category 5 or higher for a Local Area Network cable.

- RS-232C cable should be a cross type cable.

# 2	Installation

## 2.1	System Requirements

The following is the minimum system requirements of hardware and software to use the Projector Management Application.

- Operating System
Windows 2000 Professional SP3 or later version
Windows XP Professional SP1or later version

- CPU
PentiumⅢ 500Mhz or higher

- Memory
256MB or higher

- Hard disc
Minimum 10MB free space for installation

- Local Area Network port (RJ-45) or Serial port (RS-232C)

## 2.2	Projector Management Application Installation

The installation process of Projector Management Application is as follows.

- Extract the download file.

- Double click the file “SETUP.EXE”.
 Projector Management Application setup wizard appears.

- Click [Next]

- Please read license agreement, and click [Yes] when you agree.
Otherwise, click [No] (the setup wizard is then terminated).

- Select an install folder, then click [Next],

- Confirm the program folder, and then click [Next].
 Following screen appears, and starts the installation process.

- Click [Finish] to complete the Projector Management Application setup wizard.

# 3	Operating Instructions

## 3.1	Start Application

- Click [Start] on Windows.

- Select [Program] - [Projector Management Application] - [PJMan3].
 Projector Management Application starts, the main screen appears.

## 3.2	Quit Application

Select [File] - [Exit] to quit.

It is also possible to quit this software by clicking [X] on the top right corner of the window.

When the confirmation dialogue appears, select [Yes] to finish.

## 3.3	Main Screen

- Menu
the functions of the main window are as follows.

[File] menu

Exit 	--- Quit application.

[Edit] menu

Add 	--- Add a group or projector.

Group	--- Add a group.

Projector	--- Add a projector.

Delete 	--- Delete the selected group or projector.

Edit 	--- Change the settings of the selected group or projector.

[Status] menu

Refresh	--- Update the status of the selected group or projector.

[Control] menu

Power ON 	--- Turn on the selected projector power.

Power OFF	--- Turn off the selected projector power.

[Schedule] menu

Start	--- Activate scheduled event monitoring.

Stop		--- Stop scheduled event monitoring.

[Option] menu

Auto Refresh Setting 	--- Sets the interval time for monitoring.

Mail Setting 		--- Popup the e-mail setting dialogue.

[Help] menu

About 	--- Displays version information.

- Toolbar
The function of these icons are as follows.

--- Add a group

--- Add a projector

--- Delete a group or projector

--- Edit the configuration of a group or an individual.

--- Refresh status

--- Power ON

--- Power OFF

--- Start scheduled event monitoring

--- Stop scheduled event monitoring

- Status Tab

    - An individual projector

Projector Information --- Displays information regarding the selected projector

Projector Name	--- Projector name

Model 		--- Model name

Connection 		--- Connection information (IP address and MAC address / COM port)

Power Status	--- Power status (ON / OFF)

Input Source	--- Current input source

Error Status	--- Error status (Normal, Lamp error, Fan error, etc.)

Lamp Time	--- Current Lamp time in hours

Filter Time 	--- Current filter time in hours

    - A group of projectors

Group Information --- Displays information regarding the selected group.

Group Name 		--- Group name

Number of Projectors	--- Number of projectors in the group.

Comment 			--- Comment (unused)

Name 	--- Projector name

Connection 	--- IP address / COM port

MAC Address --- MAC address

Power 	--- Power status (ON/OFF)

Input 		--- Current input source

Status 	--- Error status (Normal, Lamp error, Fan error, etc.)

Lamp Time 	--- Current Lamp time in hours

Filter Time	--- Current filter time in hours

- Control Screen

    - An individual projector

Projector Information --- Displays information regarding the selected projector

Projector Name	--- Projector name

Model 		--- Model name

Connection 		--- Connection information (IP address and MAC address / COM port)

Power 	--- Turn on/off the projector power

Input Source	--- Change input source

Aspect 	--- Change aspect ratio

Whisper	--- Change whisper

Mirror		--- Change mirror status

Blank 	--- Change blank

Mute 		--- Change sound mute status

Bright		--- Change bright

[-]:Decrement value

[+]:Increment value

Contrast	--- Change contrast

[-]:Decrement value

[+]:Increment value

Sharpness	--- Change sharpness

[-]:Decrement value

[+]:Increment value

Color		--- Change color

[-]:Decrement value

[+]:Increment value

Tint		--- Change tint

[-]:Decrement value

[+]:Increment value

Auto Adjust 	--- Execute auto image adjustment

Lamp Time Reset	--- Reset the lamp time

Filter Time Reset	--- Reset the filter time

    - A group of projectors

Group Information --- Displays information regarding the selected group

Group Name 		--- Group name

Number of Projectors	--- Number of projectors in a group.

Comment 			--- Comment (unused)

Power --- Turn on/off a group of projectors power

- Schedule Screen

    - An individual projector

Projector Information --- Displays information regarding the selected projector

Projector Name	--- Projector name

Model 		--- Model name

Connection 		--- Connection information (IP address and MAC address / COM port)

Month column	--- Month

Year column		--- Year

Today column	--- Today

Time column 	--- Current

Calendar 		--- Shows scheduled events

Turquoise blue: Default schedule event

Green: weekly event

Blue: daily event

Red: Selected dates

[<<]--- Go to previous month

[>>] ---Go to next month

Schedule list	--- A list of events at specified day, a day of week or everyday (default)

[Delete] --- Delete an event from the scheduled events list

Schedule Set --- An event information.

Time 		--- Activation time

Control 	--- Execute command

[Set] 		--- Add an event to list.

    - a group of projectors

Group Information --- Displays information regarding the selected group

Group Name 		--- Group name

Number of Projectors	--- Number of projectors in a group.

Comment 			--- Comment (unused)

Month column	--- Month

Year column		--- Year

Today column	--- Today

Time column 	--- Current

Calendar 		--- Shows scheduled events

Turquoise blue: Default schedule event

Green: weekly event

Blue: daily event

Red: Selected dates

[<<]--- Go to previous month

[>>] ---Go to next month

Schedule list	--- A list of events at specified day, a day of week or everyday (default)

[Delete] --- Delete an event from the scheduled events list

Schedule Set --- An event information.

Time 		--- Activation time

Control 	--- Execute command

[Set] 		--- Add an event to list.

## 3.4	Add a group of projectors

You can define a group of projectors into the tree view. Select [Edit] -> [Add] -> [Group] to add a group.

Group Name --- Enter a group name up to 256 characters.

## 3.5	Add an individual projector

You can define an individual projector into the tree view. Select [Edit] -> [Add] -> [Projector] to add a projector.

Projector Name 	--- Enter a projector name up to 256 characters.

Model		--- Select a model.

Connection		--- Select a connection method.

RS-232C	--- Select a COM port.

NETWORK	--- Enter IP address of projector in dot notation (ex. 192.168.1.10) and enter port number.

The maximum number of projectors is 256.

## 3.6	Delete a group or individual projector

You can delete a group of projectors or an individual projector. Click a group or projector in the tree view, select [Edit] -> [Delete]. You can also delete a group or projector by clicking delete icon on the toolbar. When the confirmation dialogue appears, select [OK] to delete.

## 3.7	Change a group or individual projector settings

You can change a group or individual projectors settings. Click a group on the tree view, select [Edit] –> [Edit Group]. Or Click a projector, select [Edit] -> [Edit]. You can also change settings by clicking edit icon on the toolbar.

“Edit Projector” dialogue is as follows.

## 3.8	Update status

Select [View] -> [Refresh] to update status of a group or individual projector. This function only works when the status tab is displayed.

## 3.9	Turn on the power

Select [Control] -> [Power ON] to turn on the power. You can also turn on the power by clicking the power on icon on the toolbar. If you select a group of projectors, all projectors in the group are powered on.

## 3.10	Turn off the power

Select [Control] -> [Power OFF] to turn off the power. You can also turn off the power by clicking the power off icon on the toolbar. If you select a group of projectors, all projectors in the group are powered off.

## 3.11	Activate monitoring scheduled events

Select [Schedule] -> [Start]. You can also activate monitoring events by clicking the schedule start icon on the toolbar. You cannot add, delete or edit an event during while the event monitoring process is active. Please see the chapter 5 for the details.

## 3.12 	Stop monitoring scheduled events

Select [Schedule] -> [Stop]. You can also stop the monitoring events by clicking schedule stop icon on the toolbar. Please see the chapter 5 for the details.

## 3.13	Polling Interval Period

Select [Option] -> [Status Check Interval], enter the polling interval time in seconds.

The polling interval time must be within the range from 10 to 3600 seconds.

## 3.14	Mail Setting

A notification mail will be sent when any error or selected condition occurs. Select [Option] -> [Mail Settings] to open the mail settings dialogue.

- 3.14.1 Conditions

(1)	Send Mail 			--- Enable / Disable send mail function

(2)	Lamp Time Alarm 		--- Lamp time alarm. Enter the time to send a mail.

(3)	Filter Time Alarm 		--- Filter time alarm. Enter the time to send a mail.

(4)	When Error Occurred	--- Enable / Disable error condition mail

Select Errors: mark a check box to send an e-mail for the selected error condition.

[…]: Message setting dialogue. Enter a description for the message dialogue.

- 3.14.2 Settings

(1)	SMTP Server 	--- Enter the SMTP Server.

(2)	From Address	--- Enter the sender’s mail address

(3)	Send Address 	--- Enter the receiver’s mail addresses up to 10 addresses.

[Add]: Add a receiver’s mail address

[Delete]: Delete a receiver’s mail address

[Edit]: Edit receiver’s mail address

(4)	Subject 		--- Enter the subject

(5)	Attach File 		--- Enter the file name of attached file (optional).

[Browse]: File dialogue appears.

(6)	Mail Send Test 	--- Send a test mail

- 3.14.3 Message Setting Dialogue

Message Setting dialogue is as follows.

Enter the sentence of mail, up to 256 characters.

- 3.14.4 Mail Address Setting

Mail Address Setting dialogue is as follows.

Mail Address --- Enter the mail address to be sent, up to 256 characters.

[TO][CC][BCC]: Select a type of mail.

## 3.15	About Dialogue

Select [Help] -> [About] to open the about dialogue.

# 4	Mail Feature

## 4.1	Mail Format

The Projector Monitoring Application sends an e-mail by following format.

Time : YYYY/MM/DD hh:nn:ss

Projector Name : xxxxxx

Connection : xxxxxx

Message : xxxxxxxxxxxxxxxxxx (Typed in text on Message Setting screen)

(Sample 1)

Time: 2003/11/11 23:12:22

Projector Name: Projector1

Connection : 192.168.1.10 (00-00-87-16-8E-90)

Message: Fan Error!!!

(Sample 2)

Time: 2003/11/12 20:00:17

Projector Name: Projector2

Connection : COM1

Message: Lamp Error!!!

## 4.2	Retry Processing

The Projector Management Application retries 3 times to send an e-mail by 5 minutes interval, if the sending e-mail process is failed. After 3 times failure, the software creates an error log file “YYYMMDD.log” (YYYY: Year, MM: Month, DD: Day) in the same folder where this software are installed.

## 4.3	Others

This software does not support POP before SMTP (which does user certification before sending email). If your email server requests user certification, the e-mail feature will not work.

The e-mail sending process is triggered by the transition from normal status to error (or event) mode. When an error happens at the startup, this Projector Management Software application will not send an e-mail.

# 5	Scheduling Feature

## 5.1	Event Type

There are 3 types of event (daily, weekly, individual event).

- Daily Event
This is a daily event. The daily event is activated on a daily basis. This type of event has a lowest priority. So it’s overridden by weekly and individual event that is scheduled for the same day.

- Weekly Event
This is a weekly event. The weekly event is activated on the specified day of the week. This type of event has a moderate priority, so it’s overridden by individual event that scheduled for the same day.

- Individual Event
This is an individual event. The individual event is activated on the specified day. This type of event has a highest priority and will override a Daily or Weekly event.

Below is an example of when multiple events are scheduled. In this case, on April 29th, the individual event has a highest priority to execute, instead of weekly and daily event.

## 5.2	Add an Event

1. Click a specified day, a day of week or default (daily) on the calendar.

2. Pick a time and control function, then press [Set].

The power on/off and select source events are supported for scheduling feature.

## 5.3	Delete an Event

1. Click a specified day, a day of week, or default (daily) on the calendar.

2. Choose an event, and then press [Delete].

You cannot delete an event if it’s defined before today.