# RS232C Control Codes: PE8700

Baud Rate: 115200 or 9600

Parity: None

Data Bits: 8 Bits

Stop Bits: 1 Bit

Flow Control: None

| Code | Function |
| --- | --- |
| X01 | Power On |
| X02 | Power Off |
| X03 | Message On |
| X04 | Message Off |
| X10 | Menu (arrow key) |
| X11 | Enter (arrow key) |
| X12 | Exit (arrow key) |
| X13 | Up (arrow key) |
| X14 | Down (arrow key) |
| X15 | Left (arrow key) |
| X16 | Right (arrow key) |
| X20 | Switch to Composite Input |
| X21 | Switch to S-video Input |
| X22 | Switch to Component Input |
| X23 | Switch to D-Sub_PC Input |
| X24 | Switch YPbPr Input |
| X25 | Switch to BNC_PC Input |
| X35 | Anamorphic |
| X36 | Standard |
| X37 | Letter Box |
| X38 | Virtual Wide |
| X40 | Load Memory 1 Settings |
| X41 | Load Memory 2 Settings |
| X42 | Load Memory 3 Settings |
| X46 | Load Default of Current Source |
| X47 | Save Memory 1 Settings |
| X48 | Save Memory 2 Settings |
| X49 | Save Memory 3 Settings |
| X55 | Switch Active Source |
| X56 | Picture in Picture Display |
| X57 | Picture in Picture Display |
| X60 | Switch Language - English |
| X61 | Switch Language - French |
| X62 | Switch Language - Spanish |
| X85 | PC Input - Auto |
| X90 | Image Orientation - Floor Front |
| X91 | Image Orientation - Ceiling Front |
| X92 | Image Orientation - Floor Rear |
| X93 | Image Orientation - Ceiling Rear |
| X94 | Back Light On |
| X95 | Back Light Off |