TLC 16 User Manual Addendum

TLC 16 Firmware Release 1.01, April 28, 1995

Introduction

Firmware release 1.01 adds the following new features:

1:  Channel intensity adjustment in patterns  (Dimmable Patterns)

2:  Pattern linking

Both features are geared towards the advanced user and are therefore disabled when shipped.  Use the following configuration DIP switches to enable each feature:

DIP Switch 6 On (away from the numbers):  Enable dimmable patterns

DIP Switch 7 On (away from the numbers):  Enable pattern linking

Programming levels into a pattern DIP switch 6 On

While in pattern programming mode, touch and hold any latch pad and use the master up and down pads to adjust the level of that channel.  The dual LED character display will show the current level of the channel and while the latch pads are held will flash to indicate that the displayed value is not the master level.  More than one channel can be adjusted at a time by touching more than one latch pad.

If the channel level is fully on or off, then touching its latch pad will toggle the channel from full on to full off and visa versa, as normal.  If a channel has been set to a level other than full on or off, no toggling will occur.  Instead, the LED display will show the current level of that channel so adjustments can be made.

To release a dimmed latch during programming, bring its level to 0.

Pattern linking DIP switch 7 On

Pattern linking allows a pattern pad to activate a linked list of patterns.  Each step of the link can be any of the 16 patterns and can run from 1 to 15 times.  There can be up to 16 steps in a link.  To activate a linked list of patterns from a pattern pad, touch and hold the pad the same way you would if you wanted to override the fade rate (that feature is replaced by this one when linking is active).  A pattern or linked list will not start to run until the pad is released.  This way the pad can be used to activate either a pattern or a linked list.

To program the list, tap the program pad so that its LED is flashing.  Touch a pattern pad.  The program LED stops flashing and remains on.  The pattern LED flashes quickly to indicate that link programming is active instead of normal pattern programming.  The latch and bump pads are used to determine which pattern and how many times it loops for each step of the link.  They do not affect the stage (blind).  The latch pads are used to select the pattern for the step and bumps 1-15 are used to select the number of times that pattern will loop within that step.  Touch the program pad to save the step.  Use the blackout pad to end programming.