Club/Mobile Mixers

PRO CM1000

- INPUTS: 12 line, 4 phono, 2 mic

- OUTPUTS: Master, 2 zone, booth

- Balanced and unbalanced outputs for master, zone 1, & booth

- Optical VCA crossfader w/ slope control and fader start

- 4 Band EQ w/ rear panel sensitivity adjust and disable

- Split/blend headphone output assignable to the booth

- Process loop, light output, master gain reduction, meter sensitivity adjustment

PRO CM1000R

- Rotary version of the CM1000

EM460 w/KAOSS PAD

- INPUTS: 7 line, 3 phono, 1 mic input w/EQ

- OUTPUTS: Master balanced, 2 zone, record, MIDI, Fader-Start

- KAOSS PAD:60 effects, Sampler(5 sec/48 kHz), MIDI, real-time processing

- Front panel Neutrik™ mic input w/ bass-treble-talkover control

- 3-band rotary –30 dB cut EQ w/ pan on every channel and master

EM360 w/KAOSS PAD

- INPUTS: 6 line, 3 phono, 1 mic input w/EQ

- OUTPUTS: Master balanced, zone, record, MIDI, Fader-Start

- KAOSS PAD:60 effects, Sampler(5 sec/48 kHz), MIDI, real-time processing

- 3-band rotary EQ, Panning

CM100

- INPUTS: 8 line, 3 phono, 2 mic

- OUTPUTS: Master balanced, zone, cue, record

- Split or Blend cueing, PFL gain adjust w/ meter

- ± 6 or 12 dB 5-band graphic EQ w/ rear panel defeat

- Mic 1 w/ individual EQ and auto-mute w/ sensitivity adjustment

- Fader-start, panning on master fader

DM1200

- INPUTS: 8 line, 3 phono, 2 mic w/EQ

- OUTPUTS: Master balanced, zone, send, record

- Dual 6-band EQ, sends on each channel

- Replaceable assignable crossfader

DM1295 w/sampler

- INPUTS: 8 line, 3 phono, 2 mic w/EQ

- OUTPUTS: Master balanced, zone, send, record

- SAMPLER: 110 seconds, 4 Banks of retained memory, polyphonic playback

- Dual 6-band EQ, sends on each channel

DM1090X

- INPUTS: 6 line, 2 phono, 1 mic w/EQ

- OUTPUTS: Master, record

- Replaceable crossfader

- 5-band graphic EQ

Scratch Techno Mixers

MATRIX 3K “Fusion Design”

- INPUTS: 3 phono, 6 line, 1 mic input w/ EQ

- OUTPUTS: Master balanced, zone, record, fader start, PFL sampler send

- Omega™ true optical crossfader w/ extra wide knob, assign, slope, "hamster" switch, and action, action light, fader start

- Dual BNC light connectors, recessed rear panel

- Process looping w/ blend control

- Punch IN/OUT transform buttons

- -40 dB KILLS for bass, middle, and treble

- Channels contain: long-life input faders, 3-band rotary –30 dB EQ cuts, process insert loops, gain, panning, 8 position input toggles, PFL meters

- ¼” & 1/8” headphone jacks w/ split or blend cueing

MATRIX 2 “Fusion Design”

- 2 channels with Gain, EQ, and PFL meters

- Fader Start crossfader w/ extra wide knob

- Unique fader position light

- Recessed rear panel

- ¼” & 1/8” headphone jacks

EM260 w/KAOSS PAD

- INPUTS: 4 line, 2 phono, 1 mic input w/EQ

- OUTPUTS: Master, record, MIDI, Fader-Start

- KAOSS PAD:60 effects, Sampler(5 sec/48 kHz), MIDI, real-time processing

- Crossfader w/ hamster switch-slope adjustment and fader start

- Each channel contains : 3-band rotary –30 dB EQ cuts, gain, PFL meter

PRO SM-3

- INPUTS: 3 phono, 6 line, 1 mic input

- OUTPUTS: Master balanced, zone

- Infinity™ true optical crossfader w/assign, slope & "hamster" control

- Channels contain: long-life input faders, 3-band rotary –40 dB EQ KILLS, process insert loops, gain, panning, 8 position input toggles, PFL meters and input fader 3 position slope adjust and hamster switch

- ¼” & 1/8” headphone jacks w/ split or blend cueing

PRO SM-2

- INPUTS: 4 phono, 2 line, 1 mic

- OUTPUTS: Master balanced, zone, record

- Infinity™ true optical crossfader w/ slope & “hamster” control

- Channels contain: long-life input faders, 3-band rotary –30 dB EQ cuts, gain, panning, 8 position input toggles, PFL meters, 3 position slope

- ¼” & 1/8” headphone jacks w/ slider cueing

PRO SM-1

- INPUTS: 4 phono, 2 line, 1 mic

- OUTPUTS: Master balanced, zone, record

- Replaceable VCA crossfader w/ slope & “hamster” control

- Channels contain: 3-band rotary –30 dB EQ cuts, gain, panning, 8 position input toggles, PFL meters and input fader 3 position slope adjust

- ¼” & 1/8” headphone jacks w/ slider cueing

DM3002X

- INPUTS: 3 phono, 3 line, 1 mic w/EQ

- OUTPUTS: Master, Record, PFL cue send for sampling, fader start

- Bass, Middle, and Treble KILL switches,

- Unique Punch IN/OUT transform buttons, Panning

- Channels contain: gain, 3-band rotary EQ, PFL meter, post process loops

- Assignable crossfader w/ hamster & slope adjust

DM3001X

- INPUTS: 3 phono, 3 line, 1 mic w/EQ

- OUTPUTS: Master, Booth, Record, PFL cue send for sampling

- Bass, Middle, and Treble KILL switches

- Unique Punch IN/OUT transform buttons, Panning

- Channels contain: gain, 3-band rotary EQ

- Assignable crossfader w/ hamster and slope adjust

DM3000X

- INPUTS: 3 phono, 3 line, 1 mic w/EQ

- OUTPUTS: Master w/pan, Record, PFL send for sampling, fader start

- Channels contain: gain, 3-band rotary EQ

- Assignable crossfader w/ hamster, and slope adjust

DM1002X

- INPUTS: 2 line, 4 phono, 1 mic w/EQ

- OUTPUTS: Master, record

- Channels contain: PFL gain, 3-band rotary –30 dB EQ cuts

- -35 dB KILLS switches for bass, mid, & treble, transform buttons

- Replaceable crossfader w/ slope & “hamster” control

DM1001X

- INPUTS: 2 line, 4 phono, 1 mic w/EQ

- OUTPUTS: Master, record

- Channels contain: PFL gain, 3-band rotary –30 dB EQ cuts

- Transform buttons

- Replaceable crossfader w/ slope & “hamster” control

DM905

- INPUTS: 2 line, 2 phono, 1 mic

- OUTPUTS: Master, record

- Compact design, slider cueing

- Replaceable crossfader

RACK MOUNT CD Players

CDN 88

- Dual rack mountable CD Player

- Real time scratching, Reverse, up to ±%100 pitch

- KEY lock (master tempo w/ key control) up to 100%!

- Built in Beatkeeper w/ auto-mixing and synchronization

- 2 complete seamless loops w/ 3 hot stutter starts

- Many DSP interactive wheel effects

- ± 6,12,25 and 100% pitch control

- User programmable custom feature set

- Fader and relay start

- 48 seconds of buffer memory for Anti-Shock™

- Digital outputs, CDR compatible

CDN 33 “Fusion Design”

- Dual rack mountable CD Player

- 20 seconds Anti-Shock™, True buffered instant start

- Interactive Pitch/Search wheel with finger grip

- ±8/16% pitch, Sleep mode, BPM counter

- Light up Play/Cue buttons

- Direct track access, Track sequence program

- Additional rotary track access

- Transport bay illumination light,  CDR compatible

- Relay play, 6-way Fader/Remote start/pause or start/recue

- Digital output, Power switch protector

CDN 25 “Fusion Design” (CDN 18 EUROPE)

- Dual rack mountable CD Player

- Interactive Pitch/Search wheel with finger grip

- ±8/16% pitch, Sleep mode,  CDR compatible

- Power switch protector, All rubber buttons

- Digital output

CDN 15  “Fusion Design”

- Single rack mountable CD Player

- Interactive Pitch Bend/Search wheel with finger grip

- +/-8 or 16% pitch ranges

- Sleep mode to save laser life

- Direct Track access with sequence programming

- Power switch protector, All rubber buttons

- Transport bay illumination light,  CDR compatible

MP3 CD PLAYER

MPCD3 “Fusion Design” MP3

- Plays MP3 files from CDR and audio CDs

- 999 track index

- Single rack mount CD/MP3 player

- Direct track access with sequence programming

- Interactive Pitch Bend/Search wheel with finger grip

- +/-8 or 16% pitch ranges

- Digital output

TABLE TOP CD Players

AXIS 2 “Fusion Design”

- Tabletop CD Player

- Large Interactive Pitch wheel, Long Pitch slider

- 10 Seconds Anti-Shock™, True buffered instant start

- ±8/16% pitch, Sleep mode, Digital output

- BPM counter, Separate Relay play

- Recessed rear panel, All rubber buttons

- CDR compatible

- Relay play, 6-way Fader/Remote start/pause or start/recue

AXIS 8 “Fusion Design”

- Real time scratching on 130mm jog wheel

- Direct access of 12 DSP effects

- ± 6,12,25 and 100% pitch control on 100mm fader

- KEY lock at any tempo up to ±100%!

- Built in Beatkeeper w/ auto-synchronization

- 2 complete seamless loops w/ 3 hot stutter starts

- Reverse and Key Change

- Adjustable Startup and Braking speeds

- MIDI in/out for effects, clock, interlocks, start/stop

- User programmable custom presets

- 6-way fader/remote start/pause or start/recue & relay

- 48 seconds of buffer memory for Anti-Shock™

- Recessed rear panel with digital outputs

- Transport bay illumination light

- Never Obsolete - Open Architecture allows the user to update unit software via CDR

MP3 Controller

DMC-1 Controller w/ PCDJ Blue Software

- Familiar dual CD style design MP3 controller, pitch control, full music library access, search and sort features by various subgroups, auto play and auto pilot functions, exact cueing to multiple cue points w/in a fraction of a millisecond, multiple cue, point access, true instant start, auto beat matching, forward & reverse searching, CD ripper w/ jitter correction

PCDJ RED Software

- Auto BPM Detection, Auto Gain Normalization, Synchronized Auto-Mix, Looping, Can be used w/ DMC-1 controller, Recording, Ripping, Turntable Brake, Password Protected Record Case, 20 cue points, Beat Stepping, Proprietary MP3 File Encryption, Play list log file

Turntables

TTX (Fusion Design)

- Direct Drive Ultra High Torque Open Motor (+3.7 kgf-cm Peak)

- Straight & S-Shaped tone arms included

- “Solid-Core” Steel Top, Rubber Base

- Interchangeable Pitch Fader & Button Cartridges for club & battle style

- Detachable Power & Audio Connections

- Removable Target Light with Super-Bright White LED

- Blue Illuminated Display w/ 10,20 & 50% Pitch Range, 33-45-78 RPM, reverse, adjustable brake & startup times

- Digital out, Line Output, Key Lock

TT200 (Fusion Design)

- Direct Drive Super High Torque Motor

- Straight & S-Shaped tone arms

- Battle & Club style design

- Steel Top, Rubberized Base, Aluminum Platter

- User-replaceable Pitch Fader, Removable Aluminum Light

- Detachable Power & Audio Connections

- 33-45-78 RPM, Reverse

TT1650 (Fusion Design)

- Direct Drive High Torque Motor

- Battle & Club style design,

- Removable Target Light

- Aluminum Platter

- Detachable Power & Audio Connections

- 33-45 RPM

TT1600 (Fusion Design)

- Efficient Belt Drive Motor

- Battle & Club style design,

- Detachable Power & Audio Connections

- 33-45 RPM

Cartridges

CC-1

- Plug in Silver Design

- Ideal for general use

- 18 grams

CS-1

- Plug in Design with Rubber Grip handle

- Ideal for Club use

- 18 grams

CX-1

- Plug in Rugged design

- Silver with Blue tip

- Ideal for Scratch DJ use

- 18 grams

PHAT TRAK

- Plug in design

- Unique illuminating light

- Automatic light shut-off to preserve battery life

GROOVE TOOL

- .6 mil spherical diamond tip cartridge w/ 2.5g to 3.5g tracking force

WAX RIDER

- .6 mil spherical diamond tip cartridge w/ 2.5g to 4.5g tracking force

Complete Packages

FUNKMASTER FLEX v.2 (2002 edition)

- (2) TT1550 Multi Directional Efficient Belt Drive Turntable

- DM950 2 Channel + mic mixer, EQ on each channel, Slide Style Cueing

- HF125 Flexible 7-Position Dual Headphone

BATTLE PAK v.4 (2002 edition)

- (2) TT1625 Multi Directional Direct Drive Turntable with Aluminum Platter

- DM950 2 Channel + mic mixer, EQ on each channel, Slide Style Cueing

- HF125 Flexible 7-Position Dual Headphone

PHAT PAK

- 1 DM1001EX mixer

- 2 TT-100 turntables

- 1 set M-1 speakers

- 1 pair D-50 headphones

- Includes cartridges & slipmats

Fusion 222 (Fusion Design)

- (2) AXIS 2 Professional Tabletop CD Players with Anti-shock

- MATRIX 2 Professional 2 Channel DJ Mixer with fader start

- Semi-hard Nylon FUSION Equipment Case with shoulder strap

- All Necessary Cables and Manuals

Fusion Tabletop CD System Case

- Fusion Series CD-mixer case

- Semi-hard Nylon design

- Easy Carry Shoulder Strap

- Vanity Shield to hide cables

- Easily Removable Zipper lid

- Rear and Bottom Cable Access Shields

- Extra Front Pocket for Accessories

CD Mixer Systems

CD MIX-2

- Dual CD player and mixer combination

- Buffered Memory  Anti-Shock™ , Pitch Wheel

- Seamless looping, \Fader-start, Track programmable, ± 12% pitch,  true continuous play

- External inputs for 3 line, 2 mic, 2 phono

- Balanced output, master EQ, stereo/mono control

- EQ on each channel

CD MIX CASE

- Carpet case for transporting your CD Mix Station

Amps

DIMENSION AMPLIFIERS FEATURE:

- “Fusion Design”

- Front panel volume control w/ output meter display and clip indicators

- 5-way binding banana plug and 3 speakon outputs for left/right/bridge w/ Recessed rear bridge control switch

- Balanced XLR balanced and ¼”(6.3 mm) input w/ XLR balanced outputs (thru) for daisy chain

- Crossover for subwoofer w/ 20-200 Hz frequency adjust and 30 Hz and 50 Hz low frequency  roll off filters

- Internal compression limiter, Electronic protect for DC output and short circuits

- 2X dual speed fans coupled w/ heatsink cooling w/ Rear panel reset button

Dimension 4

1900W RMS @ 4 Ohms/ 1350W RMS @ 8 Ohms (mono bridged); 850W RMS @ 2 Ohms/ 660W RMS @ 4 Ohms/ 412W RMS @ 8 Ohms (stereo)

Dimension 3

1300W RMS @ 4 Ohms/ 930W RMS @ 8 Ohms (mono bridged); 650W RMS @ 2 Ohms/ 475W RMS @ 4 Ohms/ 300W RMS @ 8 Ohms (stereo)

Rack Accessories

RC-8 Rack Conditioner

- Voltage display w/ line conditioner

- 8 rear outlets w/ extra wide design for power adapters

- Front panel pull out lights w/ dimmer and separate switch

RM-6 Rack Mixer

- 6 line, 1 phono, 3 mic inputs

- All inputs assignable to 2 Zone outputs

- 1 Front panel mic w/ EQ and auto-mute control

- 3-Band EQ, Gain, and Panning for each zone

- Headphone output w/ volume

- Balanced and unbalanced outputs

Wireless Mics

WS31

- Single non-diversity

- 1 handheld microphone w/ mute switch

WS31 BP

- Single non-diversity

- Includes Headset, Lavaliere, & Belt pack

WS33

- Single Diversity

- 1 handheld microphone w/ mute switch

WS33 BP

- Single Diversity

- Includes Headset, Lavaliere, & Belt pack

WS 3 RK

2 system rack kit w/ cover plate

Speakers

M-40

- Self powered, 40 watts system, master/slave style

- Treble, bass & volume adjust, mounting holes

M-1BK

- Self powered monitor speaker

Headphones

D200

- Dual cup headphone

- Gold plated connectors

- Closed back

D200M

- Dual cup headphone w/ mic

- Flexible mic arm

- Gold plated connectors

- Closed back

D100

- Single cup headphone

- Gold plated connectors

- Closed back

HF150

- Dual cup foldable headphone

- Gold plated connectors

- Carry bag included

HF125

- Flexible 7-Position Independently Adjustable Dual Ear Cups

- Large 40mm Mylar Speaker Drivers

- 6ft Tangle-Resistant Cord with ¼” (6.3mm) Plug

- Padded Headband & Ear Cups

Z Headphone

- Unique small cup design

- Carry bag included

Accessories

RIGHT ANGLE ADAPTER

- Right angle plug adapter for ¼” headphone jack

Slipmat

- Available in Gold and Spiral

Pro DJ Light

- Gooseneck light for DJ mixers

Long sleeve T-shirt

- Embroidered NUMARK logo and DJ patch

Hooded Sweat Shirt

- Embroidered NUMARK logo and DJ patch

Berklee Music

Turntable Basics Book

- The fastest way to start spinning! Get the essential of playing the turntable down instantly w/ this quick-start DJ handbook. Techniques and tricks of famous DJs revealed.

Turntable Techniques Book & Record

- The first-ever turntable method! Learn to play the turntable as a musical instrument and master the latest techniques of the hottest DJs in the business.

Needle Juice

- Live hip-hop, R&B, and funk breaks & grooves: vocals & scratch substance

Turntablist Tool Kit

- Unbelievable sound FX & tones; jazzy-funky solo instruments; shout & speechifyin’.

Rockin’ the House

- Hard-core grooves, huge guitars, vocals & scratch material.

Beat Bomb

- The ultimate collection of live drum beats and loops.

Tech Tools for DJs

- Beats, scratches & sound FX for techno, trance, house, jungle & acid jazz.