MTK Protocol for Remote Control                      Rev. 3, 20060614

| # Baud Rate | 115200 |
| --- | --- |
| Data Bits | 8 |
| Stop Bit | 1 |
| Parity | None |

# Initial RS232

A sequence of 6 bytes, this command is without delay and 50h accompanying.

| ### Command |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |  |  |  | Computer to LCD TV, in Hex |
| 57600 | 02 | 00 | 04 | 13 | 01 | 00 |  |  |  |  |  |  |  |
| 115200 | 02 | 00 | 00 | 13 | 01 | 00 |  |  |  |  |  |  |  |

# Remote controller

Example for Power command, in hex, and with 50h accompanying,

50, BE, 8ms delay, 50, EF, delay, 50, 25, delay, 50, 00, delay, 50, 08

| ### Command | From computer to TV, in hex |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  | Return, from TV to computer |
| Power | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 08 |  |  | 06, 08 |
| One | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 1F |  |  | 06, 1F |
| Two | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 20 |  |  | 06, 20 |
| Three | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 21 |  |  | 06, 21 |
| Four | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 22 |  |  | 06, 22 |
| Five | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 23 |  |  | 06, 23 |
| Six | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 24 |  |  | 06, 24 |
| Seven | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 25 |  |  | 06, 25 |
| Eight | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 26 |  |  | 06, 26 |
| Nine | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 27 |  |  | 06, 27 |
| Zero | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 1E |  |  | 06, 1E |
| CH RTN | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 15 |  |  | 06, 15 |
| Sleep | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 14 |  |  | 06, 14 |
| Display | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 13 |  |  | 06, 13 |
| Mute | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 09 |  |  | 06, 09 |
| Audio | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 12 |  |  | 06, 12 |
| CH+ | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 04 |  |  | 06, 04 |
| CH- | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 05 |  |  | 06, 05 |
| VOL+ | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 07 |  |  | 06, 07 |
| VOL- | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 06 |  |  | 06, 06 |
| Menu | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 01 |  |  | 06, 01 |
| Input | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 00 |  |  | 06, 00 |
| Favorite | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 17 |  |  | 06, 17 |
| Swap | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 18 |  |  | 06, 18 |
| P.Ctrl | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 19 |  |  | 06, 19 |
| CC | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 1A |  |  | 06, 1A |
| View | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 10 |  |  | 06, 10 |
| Up | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 0D |  |  | 06, 0D |
| Down | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 0E |  |  | 06, 0E |
| Left | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 0C |  |  | 06, 0C |
| Right | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 0B |  |  | 06, 0B |
| Enter | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 03 |  |  | 06, 03 |

# Power Status

| Read Power Status | 50 | BE | 50 | EF | 50 | 25 | 50 | 80 | 50 | 08 |  |  | 06, 00 as Power OFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 01 as Power ON |

# For 42” PAL I firmware only

| Power On | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 28 |  |  | 06, 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power Off | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 29 |  |  | 06, 29 |

# Input sourcing

Example for Input-AV1,

50, 4B, 8ms delay, 50, 01

| Input-TV | 50 | 4B | 50 | 00 |  | No return |
| --- | --- | --- | --- | --- | --- | --- |
| Input-AV1 | 50 | 4B | 50 | 01 |  | No return |
| Input-AV2 | 50 | 4B | 50 | 02 |  | No return |
| Input-SV1 | 50 | 4B | 50 | 03 |  | No return |
| Input-SV2 | 50 | 4B | 50 | 04 |  | No return |
| Input-YPbPr1 | 50 | 4B | 50 | 05 |  | No return |
| Input-YPbPr2 | 50 | 4B | 50 | 06 |  | No return |
| Input-VGA | 50 | 4B | 50 | 08 |  | No return |
| Input-HDMI | 50 | 4B | 50 | 09 |  | No return |
| Input-VGA_Com | 50 | 4B | 50 | 0A |  | No return |

V2: Power On and Power Off for 42” PAL I model.

V3: Read Source and Mute status, and create more functions.

| ### Command | From computer to TV, in hex |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  | Return, from TV to computer |
| Dash | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 59 |  |  | 06, 59 |
| Freeze | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 55 |  |  | 06, 55 |
| Auto | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 11 |  |  | 06, 11 |
| Lighting | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 57 |  |  | 06, 57 |
| Aspect | 50 | BE | 50 | EF | 50 | 25 | 50 | 00 | 50 | 56 |  |  | 06, 56 |
| Read Mute Status | 50 | BE | 50 | EF | 50 | 25 | 50 | 80 | 50 | 09 |  |  | 06, 00 as Mute off |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 01 as Mute on |
| Read Source Status | 50 | BE | 50 | EF | 50 | 25 | 50 | 80 | 50 | 00 |  |  | 06, 00 as TV |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 01 as Composite 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 02 as Composite 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 03 as SV 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 04 as SV 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 05 as Component 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 06 as Component 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 07 as DTV |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 08 as HDMI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 09 as VGA |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 06, 0A as VGA_Component |